ATARI

CORPORATION

CONFIDENTIAL

TITLE

NEXT ASY.USED ON

APPLICATION

DRAWN BYDATE

CHECKEDDATE

ENGINEERDATE

APPROVALDATE

| 700065 | CD ROM |
| --- | --- |

| S.SWAMY |  |
| --- | --- |

JAGUAR CD ROM DRIVE TEST SPECIFICATION

| REV. | DESCRIPTION | DATE | APPROVAL |
| --- | --- | --- | --- |
| A | PRODUCTION RELEASE |  |  |

# Ó 1994   ATARI  CORP

# JAGUAR  CD ROM  DRIVE  MODULE

# TEST SPECIFICATION

# December  9,    1994.

# Section 1                   Overview                                      page 3

# Section 2                   In-Circuit -Test                                     page 4

# Section 3                   Static Burn-in Test                               page 6

# Section 4                  PCBA Functional Test                          page 7

# Section 5                  Final  Systems  Test                             page 12

# Section 6                  Diagnostic Test                                     page 17

# Appendix  A                 PCBA  Functional Test

# Fixture Specification                            page 21

# Appendix  B                Static Burn-in test specification          page  23

# Appendix  C                Final Systems Test  Setup drawing     page  24

# Appendix  D                Test Flow chart                                      page  25

# SECRET   AND  CONFIDENTIAL

# Ó 1994   ATARI  CORP

# 1.0    Overview

This  document  describes  various  test   procedure  and  specifications  which  must  be

run in   production.  It  is  referred  to  by  various  test  and  QA  documents  from  time

to time  including  the  acceptance  specification

Section 2   describes  the    requirements   for   in-circuit  test  -  the   primary   tool   for

identifying   and   controlling   manufacturing   costs.

Section 3   describes  the  requirements  for  Static Burn-in  test

Section 4 describes  the  requirements  for  PCBA  functional  test - designed  to  check

all  system  functions  and  provide  diagnostic  information  for  systems  which  do  not

perform  as  required.

Section 5   describes   the   requirements   for   Final   system   test - includes   electrical

mechanical   and  cosmetic  inspection  designed  to  ensure  customer  satisfaction.

Section 6   describes the tests that can be  used for diagnosis of failed PCBA’s.

Section 7    shows the complete test flow chart

Appendix A    describes the specification for  building the PCBA Functional test fixture

Appendix B    describes the Static Burn-in test specification.

Appendix C    shows  the  setup  for  final  systems  test.

# Ó 1994   ATARI  CORP

# 2.0    In - Circuit - Test

# 2.1    Equipment / Setup

# Equipment  Required:

The  manufacturer  is   responsible  for  selecting   the  equipment,  building   fixtures

and   generating    the    in-circuit  test   program. The   manufacturer   must   provide

Atari   with   a  summary  of  tests  performed  on  each   components,  including  the

level  of  coverage,  tolerances,  and   a  list  of   components   not  tested. Additional

function  tests may be  required   depending  on   the   level   of   testing  achieved  at

in-circuit test.

# Unit Under Test:

APN   700037          Jaguar CD ROM Drive Printed Circuit Assembly.

# Test Condition:

Temperature:       25 degree C  +/-   4 degree C

# Setup:

Set UUT on ICT fixture and start test,

# 2.2   Shorts Test

A shorts test  must be  performed  on  all  nodes  accessible  to  the  test  fixture. Any

node  not  included  in  the  shorts  test  must be  summarized and  reported to  Atari.

# 2.3   Analog Passive Parametric Test

A  parametric value   test   must  be  performed  on   all  analog  passive  components

individually  accessible. Values  and  tolerances  should  match  the  bill  of  materials,

Parallel  analog   passive  components  should   be   tested   in   parallel.  Any  analog

passive  components  not  tested  must  be  summarized  and  reported  to  Atari.

# 2.4   Active Devices Test

Each input pin on  active  analog  and  digital  devices  must  be  driven/state  toggled

while  the  device  is  powered   and   level/state   changes  received  on  each  device

output  pin.  Any  device  not  100% tested  must  be   summarized  and  reported  to

Atari.

# Ó 1994   ATARI  CORP

# 2.5   Frequency Test

Crystals X1 and X2 must be measured to better than 0.0025%

# 2.6   Memory Test

A CRC  and  checksum test  of  the  8bit  ROM  must  be  provided.

# 2.7   Voltage  Test

Verify  Supply  Voltage  at  TP337 (Pin 5 of U5)  and  GND (Pin 3 of U13)

=   5V DC  +/- 1%

Verify  Supply  Voltage  at  TP334 ( Pin 5 of U5) and  GND (Pin 3 of U13)

=   7V DC  +/- 1%

Verify Voltage at TP35 (Pin 9 & 10 of U8) and GND (Pin 12 of U8)

=   2.50V DC +/- 5%

Verify Voltage at TP89 (Pin 14 of U4) and GND (Pin 15 of U4)

=   1.23V DC  +/- 5%

Verify Voltage at TP88 (Pin 4 of U3) and GND (Pin 21 of U3)

=   1.35V DC  +/- 5%

Verify Voltage at TP23 (Pin 7 of U1) and GND (Pin 14 & 10 of U1)

=   2.50V DC  +/- 5%

# 2.8  Custom ASIC’s

Custom  ASIC’s  are  fully  tested  at  the  vendor. The  preferred  Atari  method  for

checking  opens  is  HP Test Jet  on  the   HP 3070.

# 2.9    Reports

A variety of weekly reports must be provided:

a.  Overall yield and first pass yield ( boards must be serialized),

b.  Failure Pareto charts identifying top 10 failing tests.

c.  Repair Pareto charts identifying top 10 component repairs.

# Ó 1994   ATARI  CORP

# 3.0   Static  Burn-in  Test

This  test  is  conducted  in  a  burn-in  rack,  with    power   applied  to   PCBA   and

without  exercising any  software.  This  test  is  conducted  on   all  production unit.

# 3.1  Equipment / Setup

# Equipment Required:

APN  700093           Interconnect  PCBA  with  5V DC Supply voltage

Power Distribution Network

Burn-in  facility

High Capacity Dual Voltage Power Supply (+9VDC,  +5VDC)

# Unit Under Test:

APN   700037          Jaguar CD ROM Drive Printed Circuit Assembly.

# Test  Condition:

Temperature :   50 degree C

# Setup:

Set  UUT  in  the  burn-in rack

Apply  9VDC  to  UUT  power  jack.

Apply  5VDC  to  UUT  through  interconnect  PCBA.

Set  burn-in  rack  temperature to  50 degree C.

# Burn-in:

The  UUT  with  9VDC  and   5VDC  power  applied,  will  remain  in  the  burn-in

rack  for  a  period  of   8hrs.   At the end of   8hrs,  the  PCBA  functional  test  is

performed  on  the  UUT.

# Ó 1994   ATARI  CORP

# 4.0    PCBA  FUNCTIONAL  TEST

This test is required on 100% of all production.

# 4.1   Equipment / setup

# Equipment required:

Manufacturer    must    provide   a   bed   of    nails   test    fixture   that  contains   a

Jaguar console  containing   test  software  and  a   known  good  CD ROM DRIVE

(CDM 12)  mechanism with  an  Atari  Test  CD  mounted. The  UUT   and   Jaguar

console  are  powered  by  Atari   AC   wall   plug  adapters. The  Jaguar  console  is

connected  to  an  IBM PC   serial  port   with  a  custom  Jaguar  serial  data    cable

software  running  on   the   Jaguar   console   starts   and   stops  the  mechanism  as

required  by  the tests. The test is  approximately 2 minutes and 20 seconds long.

APN  500498      Atari  CD  ROM  Test  Disc

APN  500273      AC  wall plug  power adapter ( 2 required )

APN  700083      Suite 2 Test  Software.

APN  500405      Custom  Jaguar serial data cable.

IBM PC

Leader LJM 1851  CD Jitter Meter

# Unit  Under  Test:

APN  700037      Jaguar  CD ROM DRIVE PCB  assembly.

# Test  Conditions:

Temperature:      25 degree C  +/-   4 degree C

Humidity:           25% to 70% relative humidity

# Setup:

Set  UUT on the test fixture

Connect UUT to Jaguar console PCBA  via CD ROM extender card.

Connect CD ROM DRIVE Mechanism to UUT.

Place Atari Test CD on the  CD ROM DRIVE Mechanism.

Connect AC adapter to UUT  power jack.

Connect AC adapter to Jaguar console PCBA.

Connect  Jaguar PCBA  to  IBM PC  serial  port  via  a custom  Jaguar

serial data cable..

Power ON to start the test software on the Jaguar Console PCBA.

The terminal will display the message

“Please wait for sanity check to complete”

Ó 1994   ATARI  CORP

If the sanity check passes the terminal will display the message

“ Quick check passed.  Starting PCBA tests “

The automated test series begins at this point

The automated test series checks the following:

EEPROM  write / read back

FFFF/0000 words serial data test

Dual counter serial data test        and

Butch FIFO to Tom 32-bit data test.

At the end of a successful test series the terminal displays the message

“ UUT self test passed   Congratulations”

“ External evaluation required for the following tests”

At the end of each measurement type any key on the PC to begin the next test.

.

# Block error rate (BLER) at  inner and outer diameters of disc       ( test # 4.4.5)

Note: To perform this test, the UUT should have the following modification

a)   The jumper link at JP225 should be between pins 2 and 3.

b)    Pin 2 of  JP 210 ( BTCH V4)  should  be  pulled high to VCC  by

a pull up resistor ( 4.7K 1/4W)

This  automated test  measures  BLER  on  the  inner  and  outer  tracks  of

the Atari  Test  Disc, at  double speed.  As  the test proceeds, the  message

BLER count was  $xxxx

This message appears 8 times

The test operator has to inpect the BLER count and determine if the

PCBA  has passed or failed the BLER test.

All BLER counts  less than $0064   ------------   PCBA passed the BLER test

Any BLER count  greater than $0064 ---------    PCBA failed the BLER test

# 4.2   Complete  System  Voltage and Current Test:

# Power Supply current ( measured at the supply point )            ( test # 4.3.1)

On the power cord, near the power input jack on the CD  ROM PCB,

measure the input current

Should be = 111 mA. +/-  10 mA

# Power Supply current draw after regulator U5 (+5V)                  (test # 4.3.2)

Measured between JP224 Pin 1(TP9) and JP224 Pin 2 (TP337)

Should be between 55 to 65 mA.

Ó 1994   ATARI  CORP

# Power Supply current draw after regulator  U13 (+7V)                (test # 4.3.3)

Measured between JP223 Pin 1(TP63) and JP223 Pin 2 (TP334)

Should be between  24  to  30  mA.

Note that this is the value “C” used below.

# Laser Current                                                                                  (test # 4.3.4)

Voltage at TP60 (R24), focus off = (your measurement A)

Voltage at TP60 (R24), focus  on = (your measurement B)

Calculate  current: Ilaser = (B-A) / 4.7

Verify that Ilaser  <  90 mA.

# Focus Current                                                                                  (test # 4.3.5)

Measure current between JP223 Pin 1(TP63) and JP223 Pin 2 (TP334)

=  ‘D’ mA

Calculate current:  Ifocus =  D - C

Verify that Ifocus < 50 mA

# Spindle motor current at double speed                                          (test # 4.3.6)

With the test CD spinning at double speed in the drive, focus and radial off,

measure current between JP223 Pin 1(TP63) and JP223 Pin 2 (TP334)

=  ‘F’ mA

Calculate current :   Ispindle 2  =  F - C

Verify that Ispindle 2 < 135 mA

# Peak Hold current                                                                             (test # 4.3.8)

As  the  test  software starts and stops the drive repeatedly, measure the peak

current between JP223 Pin1(TP63) and JP223 Pin 2(TP334).

Verify that Ipeak < 1000 mA each time.

# 4.3    Behavior of spindle motor

These tests are performed with the disc spinning at double speed.

# Spindle motor DC voltage at inner diameter of disc                        (test # 4.5.1)

Measure the  DC  voltage  between  pin 3  (TP362)  and   pin  4  (TP363)  on

JP4 (motor cable connector)  and  verify  that...

# Vmotor DC  =   1.18 Volts, +/- 20%

Ó 1994   ATARI  CORP

# Spindle motor DC voltage at outer diameter of disc                        (test # 4.5.3)

Measure  the  DC  voltage  between  pin 3  (TP362)  and   pin 4  (TP363)  on

JP4 (motor  cable  connector)  and  verify  that...

# Vmotor DC   =  0.66Volts, +/- 20%

# 4.4   HF Signal Quality Measurement:

The high frequency RF signal input to the Philips 7345 decoder  is  measured  with

a Leader  LJM 1851  CD Jitter  Meter  and  Jaguar  self  test software block  error

(BLER) counter.

Connect  Jitter  Meter   probe  to  TP34  and  ground  to  TP345  (Pin 1 of JP219)

of  the UUT.

Turn  ON  the  Jitter  Meter.   The  Jitter  Meter  setup  should  be  as  follows:

LEVEL  :    Range(Vpp )   --   1.5V

Bit   ------------   3T Bits

JITTER :     Response ------   Sigma

Range ---------    60 nS range

Polarity --------    Inverted

# I3   Voltage P-P  and  Jitter  at  disc  ID                                (test # 4.4.1 & 4.4.2)

Verify  that  I3  = 1.05 Volts P-P +/- 20%

Verify that Jre  < 32 nanoseconds

# I3   Voltage P-P  and  Jitter  at  disc  OD                               (test # 4.4.3  & 4.4.4)

Verify that I3   = 1.05 Volts P-P  +/- 20%

. Verify that Jre  < 32 nanoseconds

At  the end of  test 4.4.3  and 4.4.4   the terminal will display the message.

“UUT  test  Complete”

.            “UUT self test passed, Congratulations”

The terminal then displays the following messages

Ó 1994   ATARI  CORP

Main Menu

(B)     BLER tests, Needs JP225 and 4.7K resistor

(C)     Current and voltage tests on PCBA

(D)     Data integrity tests using Jerry , then Tom

(E)      EEPROM  write and read back test

(F)      Film clip endless-loop playback

(H)     Hole-in CD tolerance test, Needs SBC444A  CD

(J)      Jitter tests

(R)     RW  Sub codes for CD + Graphics tests

(S)      Seeks of decreasing Spans tests

Waiting......

The  test  operator  can   select  any  test  in   the  menu   by   the  letter.  Test

selection  from  the   Main  Menu  becomes   necessary  only  when  the  UUT

fails the initial test series and  the test  operator  wants to  repeat a  failed  test.

The test software in the Jaguar console runs the selected test  and the terminal

displays the Main Menu enabling the test opertor to select any other test in the

menu.

# Ó 1994   ATARI  CORP

# 5.0    Final Systems test

This  is  an   automated  functional   test series  performed  on   the   CD ROM  unit under

test.   Two  serial   data   terminals   monitor    the    test   and   displays   UUT   PASSED

or  FAILED  messages  at  the  end  of  each  test.   The test software residing  on the test

cartridges  runs  the  self tests on the UUT  and counts the Block Error rate  made by  the

UUT .  The  duration   of   the   test  series   is  about  3   minutes.

# 5.1  Equipment / Setup

# Equipment Required:

APN 800008- 001  Jaguar  game  console (2 units)

APN 700083          Suite 1 Test  cartridge A with BLER jumper.

APN 700088          Suite 1 Test  cartridge B

APN 500498          CD  ROM  test disc

APN 500405          Jaguar Serial data cable ( 2 required)

.       APN 500273          AC  Power Adapter  ( 4  required )

APN 700095          Jaguar console joystick port connecting cable.

APN 700094          Jaguar CD ROM BLER evaluation unit .

APN 501800          Jaguar  joystick port connector  with  tricolor  (red, yellow, green) L.E.D

Serial Data Terminal  ( 2 required )

Color  Television  or  Atari  SC1435  monitor  with  RGB cable.

# Unit  Under  Test:

APN  800016-001  Jaguar  CD ROM DRIVE Console

# Test  Conditions:

Temperature:      25 degree C  +/-   4 degree C

Humidity:           25% to 70% relative humidity

# Setup:

Refer setup diagram  for reference.

Mount UUT on  Jaguar game console A.

Insert the Suite 1 test Cartridge A into  the  game cartridge port of the UUT.

Mount Jaguar CD ROM BLER evaluation unit on Jaguar game console B.

Insert the Suite 1 test  Cartridge B  into  the  game  cartridge  port  of  the

BLER  evaluation unit.

Connect  joystick port 1 of Jaguar console A [OUT] and  joystick port 2 of Jaguar

console B [IN] with  joystick port connecting cable.

Connect Jaguar console A serial data port  to a serial data terminal  A  or  IBM PC  via

Ó 1994   ATARI  CORP

Jaguar  serial data cable # 1

Connect Jaguar console B serial data port to a  serial data terminal B   or  IBM PC  via

Jaguar  serial data cable # 2

Connect AC adapter to power jacks of Jaguar game consoles A and B.

Connect  AC  adapter  to  UUT  power  jack.

Connect  AC  adapter  to  CD  ROM  BLER evaluation unit  power  jack.

Lift  CD  cover  of  the  UUT , place  Suite1  CD  ROM  test  disk  and close cover.

Plug BLER signal and ground  wires from cartridge  A  into  the  BLER  measuring

terminals on the CD ROM BLER  evaluation unit.

Connect tricolor L.E.D connector  to joystick port 2  of Jaguar game console  B.

Connect Atari  SC 1435 monitor to Jaguar console A   video  port.

# 5.2   Test Description:

Power  up  the  serial data terminals A and B  and Atari SC1435 monitor.

Power  up  the  Jaguar  consoles  A  and  B  by  pressing  the  red  switch on the consoles.

The  serial data terminal B  will display

“ BLEREVAL    V2.00  15 Nov 94           Please Wait...”

“ Waiting for trigger from UUT  Joypad  port....”

The  serial data terminal A  will display

“ Please wait for sanity checks to complete ”

If the UUT  passes  the  sanity  checks  the  terminal  will  display

“ Quick-check passed.  Starting Function Tests. ”

The terminal will displays the test that is running  with passed or failed message at the end.

If  UUT passes all the  self   test, the display on the terminal  A  will look as shown below

“ Test 6.3.7  EEPROM  Write/read-back                     Passed ”

“ Test 6.3.2A  FFFF/0000  words serial data test          Passed ”

“ Test 6.3.2B  Dual-Counter serial data test                 Passed ”

“ Test 6.3.2C  Butch FIFO to Tom 32-bit data test      Passed ”

“ Test 6.3.4 Decreasing span seeks speed test               Passed ”

“  UUT  Self - tests passed    Congratulations  ”

“  External evaluation  required  for  the  following ”

“  Test 4.4.9  B.L.E.R    Requires  connection  to  Jag  tester ”

“  UUT  now  playing  track  50  ”

The   serial data terminal  B   will  display

“  Counting  Block  Errors,   Please  wait .....  ”

Ó 1994   ATARI  CORP

If  UUT  Block error  rate  is  within  the  limit  [ 64 Hex]  the terminal  B  will display the

block error  count  followed by an “OK”  message.  The  display will look as shown below.

“ BLER   count   was   $ XXXX                  OK! ”

“ BLER   count   was   $ XXXX                  OK! ”

“  UUT  PASSED,   Congratulations ! ”

The  Serial  data terminal   A   will now  display

“  UUT  played  track  OK  ”

“  Filmclip  play back  loop.  Hit any key  and it will stop at the end.”

The SC1435 monitor  connected to Jaguar console A  will  now  start  playing the movie clip.

By pressing any key on the terminal ‘A’  keyboard,  the  movie  clip will stop at the end  with

a  green  screen  background.  The  terminal  A  will  now  display.

“ Self tests  PASSED,  Congratulations ! ”

“  Jaguar  CD  Module  Functional  Test  V1.5    15  NOVEMBER   1994 ”

Main Menu

(B)      BLER tests, Needs JP225 and 4.7K resistor

(C)     Current and voltage tests on PCBA

(D)     Data integrity tests using Jerry , then Tom

(E)      EEPROM  write and read back test

(F)      Film clip endless-loop playback

(H)     Hole-in CD tolerance test, Needs SBC444A  CD

(J)      Jitter tests

(R)     RW  Sub codes for CD + Graphics tests

(S)      Seeks of decreasing Spans tests

Waiting......

The  test  operator  can   select  any   test  in   the   menu   by   the  letter.  Test

selection   from  the   Main   Menu  becomes   necessary  only  when  the  UUT

fails the  initial test series and  the  test  operator  wants to  repeat a  failed  test.

The test software in the Jaguar console runs  the  selected test  and the terminal

displays the Main Menu enabling the test operator to select any other test in the

menu.

If  BLER  test  is  selected  from  the  main menu, a  power reset  is required on

Jaguar console B  to  get the BLER  count.

Ó 1994   ATARI  CORP

Turn off  power to UUT.

Turn off  power  to  BLER  evaluation unit..

Turn off power to Jaguar consoles A and B .

Open cover of the UUT and remove CD ROM test disk.

Remove Test Cartridge from  UUT.

Remove UUT from Jaguar console.

# 5.3     Mechanical / Cosmetic  Inspection

The following  mechanical/cosmetic items  inspection  shall  be  done  at  final  test

and during the final assembly stages.

No  missing  screws

No loose internal parts

No unacceptable molding defects in plastic case

No scratches, dirt, or finger prints on case

CD ROM drive mechanism  properly clamped to PCA

Correct serialized rating label in place

Correct Laser Safety label in place

Correct Class 1 Laser label in place

PCBA properly aligned in plastic case.

No bent or warped EMI shields

EMI  Shields  properly  placed inside the case.

CD  cover shall open without any friction when eject knob is pressed.

CD cover when opened, shall stand in opened position until closed by

the user

CD shall stop spinning when CD cover is opened.

# 5.4    Packout  Inspection

All  items  listed  on  the  BOM  shall  be  included as described  on the associated

assembly   drawings.  The  gift   box  shall  be  weighed  to  check  if  any  item  is

missing  inside  the  box.

This ends the final system test.

# Ó 1994   ATARI  CORP

# 5.5   Report

Failure data shall be collected on all nonconforming units by failure code.

Repair actions associated with failure code shall also be collected. This

information shall be reported to Atari monthly. In addition, information on

top 10 test failures and top 10 repair actions shall be reported weekly to Atari

in Pareto chart format.

# 5.6   Fully Automated Functional Test Error Messages

Automated Test                                                      Failure Message or State

1.   Door open/closed switch                                       no test start

2.   Data Integrity                                                        (type) failed

3.   Audio playback                                                      DSA error condition

4.   Multiple seek speed test                                         DSA error condition

5.   Verify CIRC detection circuit                                 no SAA7345 MISC output

6.    Subcode R-W channels                                         Subcode R-W not valid

7.    EEPROM read/write                                             failed

8.    Subcode  Q                                                           no subcode Q on V4

Ó 1994   ATARI  CORP

# 6.0   Diagnostic Tests

Any  test  contained  in  the  Final  Systems  Test  can  be  executed  manually  as  a

diagnostic  test,  when  the  UUT  is  connected  to  a   serial  data  terminal  or  PC

emulating  a  serial  data  terminal.  The Static behavior of  servo test  and  the Gain

tests are not available as part of  the production tests but can be invoked to trouble-

shoot defective units in diagnostic mode. ( These tests are not required, but they are

useful in trouble shooting the PCBA.

# 6.1  Static behavior of servo drivers

These tests are performed with power on the circuit, but no disc in the drive.

# 6.1.1   Focus driver offset voltage

Measure the voltage between pin 2 (TP366) and pin 3(TP367) on JP3 (flex circuit

connector) and verify that...

Vfocus 1    = 0.00 Volts, +/- 0.450 V

# 6.1.2   Radial driver offset voltage

Measure  the  voltage between pin 1(TP369) and pin 4(TP368) on JP3(flex circuit

connector)  and  verify  that...

Vradial 1  =  0.00 Volts , +/- 0.450 V

# 6.1.3   Sled driver offset voltage

Measure the voltage between pin 5(TP364) and pin 6(TP365) on JP4(motor cable

connector)  and  verify  that...

Vsled   =  0.00 Volts , +/- 0.450 V

# 6.1.4   Spindle driver offset voltage

Measure the voltage between pin 3(TP362) and pin 4(TP363) on JP4(motor cable

connector) and  verify  that...

Vmotor 1  =  0.00 Volts , +/- 0.450 V

# Ó 1994   ATARI  CORP

# 6.2     Gain tests

Note that the following 3 gain tests can damage the circuit if run for more

than 5   seconds.

# 6.2.1     (5 Seconds only!)      Focus driver gain

Apply  3  volts  to  TP86 and measure the voltage between pin 2(TP366) and

pin  3 (TP367)  on  JP3  ( flex circuit connector )  and  verify  that....

Vfocus 2   =  0.780 Volts, +/-  20%

# 6.2.2    (5 Seconds only!)      Radial driver gain

Apply  3  volts to TP87 and measure the voltage between pin 1 (TP369) and

pin  4 (TP368) on  JP3  ( flex  circuit  connector )  and  verify  that....

Vradial 2  =  0.780 Volts,  +/- 20%

# 6.2.3    (5 Seconds only!)     Spindle driver gain

Apply 3 volts to TP84 and 2 volts to TP 85 and then measure the voltage

between pin 3(TP362 )and pin 4(TP363) on  JP4 ( motor cable connector )

and verify that....

Vmotor 2  =  0.780 Volts,  +/- 20%

.

# 6.3.1    Door Open/closed detection

The   CD  Module   top   lid   has    a   contact   switch   that   grounds   a   signal

wire  to  Butch  ( ASIC at ref loc U9 ). At  test   start,  the  lid  must   be  closed.

If  the  lid  is  not  closed   no  further   tests  may   be   conducted  on  the  UUT.

Failure Condition                                                State or error code

Lid switch not connected                                           UUT held in reset

# 6.3.2    Data Integrity

The drive is  commanded   to   read   a  series  of   tracks  on  the  disc, in  various

play command  formats.  The  track  data  are  verified  algorithmically.  Any  data

errors  cause  a  test  failure.  Any   drive  error   will   cause  a   test   failure.

Ó 1994   ATARI  CORP

Failure Condition                                                   State or error code

Drive Error                                                      DSA error condition

Data Err                                                           “failed” message

# 6.3.3   Audio Playback

The  drive  is  commanded  to  play  an   inner  audio  track, an  outer audio track,

and   intermediate   tracks   containing  intentional  errors  and  defects. The  drive

must  track  through  these  without  errors.  Any  drive  error  will   cause  a  test

failure.

Failure  Condition                                                   State or error code

Drive error                                                       DSA error condition

# 6.3.4   Multiple seek speed test

The drive is commanded to seek between a set of target -times on  the  disc  while

a  timer  is  running. The test must  complete  within the specified time, or the  test

fails.  Any  drive  error  will  also  cause  a  test  failure.

Failure condition                                                  State or error code

Drive Error                                                       DSA  error condition

Excessive time                                                    Seek timeout message

# 6.3.5  Verify CIRC detection circuit

The area  of the test disc with a known  defect is played repeatedly while the  count

of  anticipated errors is  matched  to  the  actual  count. A difference  will  cause  a

failure,  as  will a count of  zero. Any  unanticipated drive errors  will also cause  a

test  failure.

Failure condition                                                State or error code

Drive Error                                                    DSA error condition

Missed CIRC                                               MISC pin error message

# 6.3.6   Subcode R-W Channels

Subcode  from  the  R  through  W  channels  is read from a predetermined area of

the disc with a known pattern. Any deviation  from  this  pattern  causes  a  failure.

Any  drive  error  also  causes  a  failure.

Failure condition                                      State or error code

Drive error                                         DSA error condition

R-W subcode error                                   R-W subcode error

Ó 1994   ATARI  CORP

# 6.3.7   EEPROM read/write

The EEPROM on the CD Module is written with a known pattern, and  then  read

back.  Any  differences  cause  a  failure.

Failure condition                                            State or error code

IC wrong                                                      System hangs

IC missing or defective                                       “failed” message

# 6.3.8   Subcode Q

Subcodes from  the  Q  channel   is   read  from   the  disc.  If  the  subcode  is  not

incrementing  as  it  should when the disc plays, the test fails.  Any drive error also

causes  a  failure.

Failure condition                                                Extended error code

Drive  Error                                                     DSA Error Condition

Q subcode error                                                 no subcode Q on V4

Ó 1994   ATARI  CORP

# APPENDIX  A

# PCBA FUNCTIONAL TEST FIXTURE SPECIFICATION

The   bed-of-nails  ( PCBA)   functional   test   fixture  shall   incorporate   various  system  components  and  provide  connections  from  these  components  to  the Unit  Under Test (UUT) PCBA.

# 9VDC power

The UUT connector JP12 shall  connect to  a  9V DC  power  source  with  a  DC current meter ( with  peak- holding  setup  required !)  connected   in  series  with  the  positive  side  of  the  DC   input   power,  JP12   pin 2  (TP65). 9VDC  power  shall   be   provided

to  the  Jaguar  console  PCBA  by  two Atari  AC  adapters.

# 108 pin connector

The UUT 108 pin connector shall interface  to  a  Jaguar  console  PCBA,  which  will   be   part   of   the   test   fixture   and    contain    the    test    software.  This   connection  may  be  provided  to the UUT  through  a  CD-ROM  extender  card  to  J801.

# +5VDC regulator output

The UUT jumper JP224 must be left out  and  a  DC  current  meter  connected   in  series between JP224 pin 1 (TP9) and JP224 pin 2 (TP337).

# +7VDC regulator output

The UUT jumper JP223  must   be  left  out  and  a peak-holding  type  DC  current  meter  connected  in  series  between  JP223  pin 1 (TP63)  and  JP223  pin 2   (TP334).

# CDM12 motor

The  UUT  connector JP4  must  connect  to  a  CDM12  motor. The  CDM12  shall  be  a

permanent part of the  test fixture  unless  the  system  CDM12   is  installed  on  the UUT  PCBA  prior  to  test  and  shipped  as  a  complete  assembly.

# CDM12 Laser

The  UUT  connector  JP3  must  connect  to  a  CDM12  laser. The   CDM12  shall  be  a

permanent  part of  the test  fixture unless  the  system  CDM12  is  installed on the  UUT

PCBA  prior  to  test  and  shipped  as  a  complete  assembly.

Ó 1994   ATARI  CORP

# BLER

The  UUT  jumper   between   pin 1  (TP344)   and   pin 2  (TP346)  of   JP225   must   be

left  out. The  test  fixture  must   short   pin 2  (TP346)   and   pin 3  (TP343)  of   JP225.

TP 375  (CDV4 signal)   should  be  pulled  high   to  +5V with   a   4.7K  1/4W   resistor.  This  enables  the  test  software  to   measure   the   block   error  rate.

# JITTER

The  Jitter   measurement  shall  be  done  on  the  UUT for  inner  and  outer  tracks  of the CD  ROM  test  disk. Test   points  TP34  [pin 8  of  U8  (HFIN) ]  and  TP345 [pin 12  of

U8 (HFGND)]  of  the  UUT  shall  be  used   for  jitter  measurements  and  corresponding

peak   to   peak  voltage  measurement. Provision  to connect  a  CD  jitter  meter ( Leader

LJM 1851 CD Jitter Meter )  at  above  specified  test   points  shall  be  provided  on  the

test  fixture.

# VOLTAGE MEASUREMENTS

The  test  fixture  shall  have  the  capability  to  measure  voltages  at   the  following  test

points on the UUT.

DC  Voltage between TP60 (pin 17 & 18 of U4) and  GND (pin 19 of U4)

DC  Voltage between TP362 (pin 3 of JP4) and TP363 (pin 4 of JP4)

DC  Voltmeters  shall  be  attached  to  the  test  fixture to measure the voltages

at  the  above  specified  test  points.

.

Ó 1994   ATARI  CORP

# APPENDIX B

# STATIC  BURN-IN   SPECIFICATION

The  Static  Burn-in  facility  will  include  a  burn-in  chamber  operating  at  a  constant

temperature of  50 degree C  and  power  distribution  network. The  power  distribution

network  will  provide  dual  DC  supply  voltage  of  9V  and  5V  to power the  PCBA.

The  9V  supply  will  power  the  PCBA   through power  jack JP12.

The 5V  is  supplied  to the PCBA  through  an   interconnect  pcb  ( provided  by Atari)

plugged  into connector J801  of  the  PCBA.

Specification for interconnect PCB.

The   interconnect   card  used  in  static  burn  in  is  a  PCB that  can  be  inserted  into

connector  J801 of  CD ROM  player  PCBA  (700037)   and   supplies  +5V to PCBA

to  turn  ON   the  voltage   regulators  U5   and  U13.  The  interconnect  card  has  no

components.

The  +5V is  at  pin 38A  and ground is at  pin  21A  of  the adapter  card.  The  5V can

be  wired  directly   to  the  card  from   a  high   capacity   power  supply .  The  power

supply should be OFF  before inserting  the card into the PCBA  and at  the  the  end of

the test the  power supply should  be  turned  OFF  before  removing  the card from the

PCBA .