-----------------------

JagPeg Conversion Tools

-----------------------

Copyright 1993 Atari Corp. All Rights Reserved.

Author: Mike Pooler, Atari Corp.

Release Date: 8/3/93

CONFIDENTIAL.

This describes how to use version 1.0 of TGAJAG, a utility that

converts a Targa file (.TGA) into a JagPeg file (.JAG).

Background: JagPeg is our version of JPEG. A JagPeg file has a .JAG

extension, and it contains one compressed bitmap. It is uncompressed

by a program called DEJAG, which runs in the GPU. This is described

seperatly.

TGAJAG takes a Targa file as input. The Targa file must be:

- Uncompressed (Image type 2)

- 24-bits per pixel

- A mutiple of 8 pixels in width

(If HALF or QUAD subsampling, a multiple of 16 pixels in width)

(Subsampling is explained below)

- A mutiple of 8 pixels in height

(If QUAD subsampling, a multiple of 16 pixels in height)

Some Targa files which meet the above requirements are stored from bottom-up,

(with the first screen-line at the end of the file), others from top-down.

Both types are supported by TGAJAG.

TGAJAG writes a JagPeg (.JAG) file as output. The JagPeg file has this

format:

[Word 0]  JagPeg version number (currently 1)

[Word 1]  Picture format

1: YCrCb with no subsampling

2: YCrCb with HALF subsampling

3: YCrCb with QUAD subsampling

[Word 2]  Width in 8-pixel blocks (e.g. 40 == 320 pixels)

[Word 3]  Height in 8-pixel blocks

[Data...]

Using TGAJAG

------------

TGAJAG is a batch file that runs under GULAM. To run use it,

type "TGAJAG (file) [quality]" where (file) is the FIRST name

(leave off the .TGA).

TGAJAG takes the optional parameter [quality], which is a number that sets

the quality of the JegPeg that is produced. The number should be between

25 and 75.

Higher numbers give a less lossy compression, thus larger picture

sizes. Lower numbers get you better compression, but if the number is too low

you will get a visable degregation in visual quality (this will appear

as fuzzyness and/or blockyness). The goal is to find a number that

gives you acceptable compression and a picture that is is nearly

indistinguishable from the original, visually. This "ideal" setting is

different for different pictures, so it's a matter of trial and error.

The default setting is 75, which is usually a good starting point. If

you go much above 75 you lose a lot of compression without much gain in

quality. If you go below 25 you may get a picture of poor visual quality.

The range of accepted settings is 0-100 but the reasonable range is

25-75.

Each setting produces a unique "dequantize" table, which is used during

the decompression (see DEJAG.DOC).

SUBSAMPLING

-----------

You also have the option of using HALF subsampling or QUAD subsampling.

To get HALF subsampling, use TGAJAGH instead of TGAJAG

To get QUAD subsampling, use TGAJAGQ instead of TGAJAG

HALF subsampling means that the picture's color data is cut to

half its horizontal size before processing by CJPEG. While this is

quite lossy, it produces almost no visable change in the final output,

while getting a good reduction in size. We recommend that you use

TGAJAGH as default.

QUAD subsampling means that the picture's color data is cut to

one quarter its size (half vertical, half horizontal) before

processing by CJPEG. Size reductions over HALF are less dramatic, visual

differences are more dramatic. It may be worth a try, but we

recommend HALF as your best option.

Remember if you use HALF or QUAD, picture width must be a multiple of

16 pixels. If you use QUAD, picture height must be a mulitple of 16

pixels.

Notes:

You should have at least double TGA-size bytes free, to be safe, before

running TGAJAG (e.g. for a 200K TGA file have 400K free). Note that the

utilities do not detect disk-full conditions, and they will probably do

a whole lot of nothing once disk space runs out (even if they look like

they're working).

If the final JAG file is larger than you expect, try using TGAJAGH instead

of TGAJAG, and/or calling it with a lower quality setting.